Faithful Guardian Aviation

CESSNA 172N

Normal Checklist

This checklist is only for training purposes in FGA aircraft and is not intended to

replace the POHVAFM. Refer also to the POHAFM in an actual emergency.

………………Preflight Inspection…..……..…

DISPATCH IN FLIGHT CIRCLE ..........................SENT & PRINT WEATHER ................................................................. CHECK

WEIGHT AND BALANCE .........................................COMPUTE

FUEL (Full, unless limited by W&B) .............................. AS REQ OIL (4 Qt. Minimum) .................................................... CHECK PITOT HEAT .....................................................(If IFR) CHECK

INTERIOR

FIRE EXTINGUISHER ............................................. CHECK AIRCRAFT DOCUMENTS ........................................ CHECK CONTROL WHEEL LOCK ...........................................REMOVE POH/AFM…..................................................................CHECK IGNITION SWITCH ............................................................OFF MASTER SWITCH ..............................................................ON

FUEL QUANTITY ......................................................CHECK ALTERNATE STATIC SOURCE ..........................................OFF

BEACON, STROBE, NAV, & LANDING LT ...................... CHECK

FLAPS ........................................................... FULLY EXTEND MASTER SWITCH .............................................................OFF

FUEL SELECTOR VALVE ............................................BOTH BAGGAGE AND LOOSE ITEMS ................................. STOWED

EMPENNAGE

BAGGAGE DOOR …….................................................CHECK CONTROL SURFACES ............... (Freedom & security) CHECK TRIM TAB ........................................................... CHECK ANTENNAS ................................................................ CHECK

RIGHT  WING TRAILING EDGE

FLAP ……………….........................................(Security) CHECK

AILERON ......................................(Freedom & security) CHECK

RIGHT WING & LEADING EDGE

FUEL QUANTITY .........................................CHECK VISUALLY FUEL FILLER CAP ...............................................SECURE FUEL TANK SUMP ...................................................... CHECK

Check for water and sediment.

MAIN WHEEL TIRE, BRAKE DISC & PADS ................... CHECK Check for wear and proper inflation.

Continued....

_______________________________________

172 N Normal                                                                                                                         1

………Preflight Inspection (continue)……….              ………………………  …NOSE
WINDSHIELD ...............................CHECK FOR CLEANLINESS FUEL STRAINER .............................................TAKE SAMPLE    …………….     .Check for water & sediment.
OIL LEVEL ................................................................ CHECK …………Do not operate with less than 4 quarts.                                                          …………………………...Fill BELOW 4 Qts.                            OIL ACCESS DOOR .................................... .............SECURE ENGINE COOLING AIR INLETS .................................. CHECK PROPELLER & SPINNER ........................................... CHECK LANDING & TAXI LIGHTS (if applicable) ....................... CHECK ENGINE AIR FILTER ...................................................CHECK NOSE WHEEL STRUT(Approx.3"Extension) ...................CHECK NOSE WHEEL TIRE .....................................................CHECK STATIC SOURCE OPENING .........................................CHECK …………………………….       .LEFT WING                         QUANTITY ................................................. CHECK VISUALLY FUEL FILLER CAP .....................................................SECURE PITOT TUBE COVER ................................................ REMOVE PITOT TUBE ................................................................CHECK STALL HORN OPENING ...............................UNOBSTRUCTED  FUEL TANK VENT ........................................UNOBSTRUCTED FUEL TANK SUMP ...................................................... CHECK                              ……………………LEFT WING TRAILING EDGE         
AILERON ................................... (Freedom & security) CHECK FLAP ......................................................... (Security) CHECK MAIN WHEEL TIRE, BRAKE DISC & PADS .................. CHECK

…………...Before Starting Engine…….……. PREFLIGHT INSPECTION .................................... COMPLETE WT&BAL, DOCS, PERFORMANCE .............................. CHECK

PASSENGER BRIEF                       .

Doors, Seats & Seatbelts (Positive Latch),
Fire Extinguisher, No Smoking, PIC Authority, Positive Exchange of Flight Controls

SEAT & SEAT BELTS ............................... ADJUST AND LOCK CIRCUIT BREAKERS ...............................................CHECK IN ELECTRICAL SWITCHES ..................................................OFF IGNITION SWITCH ............................................................OFF AVIONICS MASTER SWITCH ............................................OFF ALTERNATE STATIC SOURCE ......................................... OFF FUEL SELECTOR VALVE  ...............................................BOTH CARB HEAT ...................................................................COLD MASTER SWITCH ............................................................. ON BEACON/ANTICOLLISION LIGHT ....................................... ON POSITIONLIGHT (night only) ................................................ON

_______________________________________   172 N Normal                                                                                                                    2

………………......Engine Start……………..…

COLD ENGINE                                     .

PRIMER ......................................... 3 STROKES & LOCK MIXTURE .............................................................RICH

THROTTLE ..................... CYCLE 1 STROKE & 1⁄4" OPEN BRAKES ...................................................... HOLD PROP AREA ............................................... CLEAR IGNITION ...........................................(8 Seconds Max) START

WARM ENGINE                                   .

MIXTURE ……......................................................RICH THROTTLE ...............................................1⁄4" OPEN BRAKES ............................................................ HOLD PROP AREA ...................................................... CLEAR

IGNITION ............................................(8 Second Max) START

IF ENGINE DOES NOT START                     .

WAIT 30 SECONDS

THROTTLE .......................CYCLE 2 STROKES & CLOSE IGNITION ...........................................(8 Second Max) START

(Do not attempt another start by cycling throttle. Use primer only.)

WHEN ENGINE STARTS                       .

THROTTLE ............................................ 1000 RPM OIL PRESSURE .......................(In 15 seconds) CHECK GREEN MIXTURE ............................... LEAN FOR RPM RISE THROTTLE .................................................... 1000 RPM FLAPS ......................................................................UP

…………………….After Start…………………

AVIONICSMASTERSWITCH …….….…......................ON HEADSETS ……........................................................ON COMS ........................ ATIS/AWOS/CLEARANCE IF REQ ALTIMETER ……......................................................SET GPS ........................... ENTER WPT / FPL / APR AS REQ. NAVS ................................................... SET / COURSE FLIGHT INSTRUMENTS

(AI/TC/COMPASS &HI/VSI)…………………………..CHECK

……………………..Taxi……………...….……

IPAD TAXI DIAGRAM…..…………………………….…..OUT

TAXI CLEARANCE .................................OBTAIN / BRIEF TAXI AREA ........................................................ CLEAR BRAKES ............................................................CHECK FLIGHT INSTRUMENTS(AI/TC/HI/VSI)..................CHECK

Check in turns

STERILE COCKPIT......................................................... ............................NO NON-ESSENTIAL CONVERSATION

_______________________________________________________________________172 N Normal                                                                                                                      3

Do not allow oil temperature to reach redline and be vigilant of oil pressure. Hold brakes securely and remain vigilant for aircraft movement; only perform runup on surface free from gravel/dirt.

………………..…….Run Up………………………

MIXTURE ......................................................FULL FWD THROTTLE ....................................................1700 RPM MAGNETOS ........................................................CHECK

(Drop should not exceed 150 RPM on either magneto or a difference of 50 RPM between the two.)

ENGINE INSTRUMENTS & AMMETER ..................CHECK SUCTION GAUGE ……...........................................CHECK CARBHEAT …………..............ON/CHECKFORRPMDROP THROTTLE .............................IDLE / CHECK 600-800 RPM CARBHEAT....................................................................OFF

THROTTLE……………………...............................1000RPM MIXTURE ................................................ SLIGHTLY LEAN

Cleaning Fouled Spark Plugs        s

THROTTLE ................................................... 2000 RPM MIXTURE ...................... LEAN FOR SLIGHT RPM DROP

MIXTURE ........................................... FULL FORWARD THROTTLE ...................... 1800 RPM FOR 60 SECONDS MAGNETO……………………………………………. CHECK

………………..…Before Takeoff………………..

PRE-FLIGHT DATA SHEET……………………………BRIEF  FLIGHT CONTROLS ..........................FREE & CORRECT FLIGHTINSTRUMENTS ……......................CHECK & SET FUEL GAUGE / QTY ......................CHECK / SUFFICIENT FUELSELECTOR..................................................BOTH TRIM ............................................................... SET T/O

GPS ..........................................(As req for departure) SET HEADING INDICATOR …….....................................SET

MAG COMPASS & HEADING INDICATOR…..…….. ALIGN

ENGINE INSTRUMENTS ...................................CHECK SEATBACKS ...................................................... UPRIGHT SEATBELT & SHOULDER HARNESS ............FASTENED

––––––––––––––––––––

DOORS & WINDOWS .......................CLOSED & LOCKED LANDING LIGHT ………...........................................ON STROBE LIGHT .................................... (If installed) ON TRANSPONDER ......................................................ALT

FLAPS ....................................................................SET

MIXTURE ..................... FULL FWD (Or set for altitude)

IPAD ............................................................................. MAP

_______________________________________________________________________172 N Normal                                                                                                                      4

_______After Takeoff (Out of 1000' AG)____

AIRSPEED .............................................................VY / VX

SPECIFY EN ROUTE CLIMB PROFILE ………..…80 KIAS Lower                              FLAPS.............................................................................UP THROTTLE……………………......................................FULL MIXTURE .......................... FULL RICH BELOW 3000' MSL

___________________Cruise____..._________

POWER .............................(≤75%PowerperPOH/AFM) SET ENGINEINSTRUMENTS..........................................CHECK

LANDING LIGHT……………………....ON (local) / OFF (xc)

MIXTURE ......................................... LEAN AS REQUIRED

MAG COMPASS / HI .................................... CHECK / SET

In-Range / Descent (XC / ACPP Only)  ..

ATIS/AWOS (As early as possible) ..........................CHECK ALTIMETER.................................................................. SET PRELIMINARY APPROACH BRIEF

Type of Approach
    Runway length / Lighting
    Field Elevation / Sector Altitude
    Highest Obstacle / Terrain Review
    NAV / Course / Minimums / Missed
 Crosswind Component…................................. COMPLETE

AIRPORT DIAGRAM (Keep available) ….…….........CHECK SEATBELTS……………………..........................FASTENED MIXTURE ........................................................... SLIGHTLY ENRICH CARB HEAT………................................ AS REQ

__Approach - Approx. 15 NM from Airport….

ATIS/AWOS…........................................................ CHECK APPROACH BRIEFING ...................................COMPLETE ALTIMETER……………................................................ SET HEADING INDICATOR TO COMPASS………..............SET FUEL SELECTOR………………................................ BOTH PARKING BRAKE ............................... CHECK RELEASED LANDING LIGHT……..................................................... ON

DIRECT IAF or VECTORED         .

FLIGHT & NAV INSTRUMENTS .......................SET/IDENT GPS (As Required for Approach)……………................ SET AIRSPEED ............................................................. 90 KIAS

_______________________________________

172 N Normal                                                                                                                                                5

Before Landing Checklist           ….

FUELSELECTOR……............................................BOTH MIXTURE ……........................................................FWD CARBHEAT ..............................................................ON

DO NOT SLIP AIRCRAFT WITH FLAPS EXTENDED

Go Around/Missed Approach       ….

THROTTLE …….....................................................FULL CARBHEAT ………...................................................OFF FLAPS ......................................... (If > 20°) RETRACT 20° PITCH ....................................................... BEST CLIMB FLAPS .......................................(>55KIAS) RETRACT 10° AIRSPEED ...........................................................VY / VX

AT SAFE ALTITUDE & >65 KIAS

FLAPS ......................................................................UP   FLY ASSIGNED/PUBLISHED HEADING & ALTITUDE

After Landing Checklist           ….

TRIM ……................................................... CENTERED CARBHEAT ………..................................................OFF FLAPS ……................................................................UP LANDING & TAXI LIGHTS .................................. AS REQ MIXTURE ............................................ SLIGHTLY LEAN TAXI CLEARANCE ............................... OBTAIN & BRIEF

Shutdown/Terminate Checklist    ….

AVIONICS MASTER………............................................OFF THROTTLE ......................................................... 1000 RPM MIXTURE ...............................................................CUTOFF IGNITION SWITCH ...................................................... OFF MASTER SWITCH .........................................................OFF ELECTRICAL SWITCHES ….........................................OFF

HOBBS/TACH…………………………………..……RECORD

FLIGHT TRACKER & MX REPORT SMS ..... SENT & ACK'D FLIGHT CONTROLS .............................................SECURE SUNSCREENS ...................................................... INSTALL TIE DOWN / CHOCK ..............................................SECURE WALK AROUND ................................................COMPLETE

______________________________________________________________________  172 N Normal                                                                                                                      6

Faithful Guardian Aviation

CESSNA 172 N

Emergency & Abnormal Checklist

This checklist is only for training purposes in FGA aircraft and is not intended to replace the POH/AFM. Refer also to the POH/AFM in an actual emergency.

Engine Failure During Takeoff Roll      .

THROTTLE …….................................................CLOSE BRAKES ……......................................................APPLY FLAPS ………............................................................UP MIXTURE ....................................................... CUT OFF IGNITION SWITCH ................................................. OFF MASTER SWITCH .................................................. OFF

Engine Failure During Takeoff       .

MAINTAIN AIRCRAFT CONTROL

LAND ON REMAINING RUNWAY OR

WITHIN 30 ̊ OF CENTERLINE. AVOID OBSTACLES.

DO NOT ATTEMPT 180 ̊ TURN.

AIRSPEED……..............BESTGLIDE/65-70KIAS(Flaps Up) FLAPS .......................................................... AS REQ. POWER .................................................AS AVAILABLE TIMEPERMITTING ....................DECLAREEMERGENCY MIXTURE ....................................................... CUT OFF FUEL SELECTOR VALVE ...................................... OFF IGNITION SWITCH ................................................. OFF FLAPS ........................(40˚/ 60KIAS Recommended) AS REQ MASTER SWITCH .................................................. OFF DOORS ........................................................ UNLATCH

_______________________________________________________________________

172 N Emergency & Abnormal                                                                                          1

Spin Recovery                 .

In the event the aircraft enters an unintentional spin, proceed as follows.

THROTTLE………....................... IMMEDIATELY CLOSED AILERONS .....................................................NEUTRALIZE

RUDDER ............................. FULL, OPPOSITE ROTATION ELEVATOR CONTROL …………...….BRISKLY FORWARD PAST CENTER ONCE ROTATION STOPS

RUDDER ........................................................NEUTRALIZE SLOWLY RECOVER FROM DIVEWHEN STRAIGHT AND LEVEL

THROTTLE ................................................................ FULL

Cabin Fire                      ..

MASTER SWITCH ........................................................OFF VENTS, CABIN HEAT & AIR .................................CLOSED FIREEXTINGUISHER .........................................ACTIVATE WHEN FIRE IS OUT

CABIN ...............................................................VENTILATE LAND AS SOON AS POSSIBLE

Wing Fire                         .

LANDING/TAXI LIGHT SWITCHES ......................OFF NAV LIGHT SWITCH .............................................OFF STROBE LIGHT SWITCH.......................................OFF

PITOTHEAT............................................................OFF

Perform a sideslip to keep the flames away from the fuel tank and cabin. Land as soon as possible using flaps only as required for final approach& touchdown

Electrical Fire                      .

MASTER SWITCH .......................................................OFF ALL ELECTRICAL SWITCHES.(Except ignition switch) OFF VENTS, CABIN HEAT & AIR ..................................CLOSED FIREEXTINGUISHER .........................................ACTIVATE

WHEN FIRE IS OUT

CABIN ...............................................................VENTILATE

IF FIRE APPEARS OUT & ELECTRICAL POWER REQ'D MASTER SWITCH ........................................................ ON CIRCUIT BREAKERS ...................... (Do not reset) CHECK MASTER AVIONICS SWITCH ...................................... ON RADIOS .............(One at a time with a delay after each) ON

LAND AS SOON AS POSSIBLE

_______________________________________________

172 N Emergency & Abnormal                                                                                         2

Engine Fire In Flight               .

MIXTURE ............................................... IDLE CUT OFF FUEL SELECTOR .................................................. OFF MASTER SWITCH .................................................. OFF CABIN HEAT & AIR (Except overhead vents) ................ OFF AIRSPEED ......................................INCREASE AS REQ TO EXTINGUISH FIRE

REEFER TO:

"EMERGENCY LANDING NO ENGINE POWER" CHECKLIST

Engine Fire During Start           .

Continue cranking in an attempt to get the engine started which would suck the flames and accumulated fuel through the carburetor and into the engine.

IF ENGINE STARTS                        .

THROTTLE ................................(For 2 minutes) 1700 RPM MIXTURE ........................................................ CUTOFF

IF ENGINE DOES NOT START             .

MIXTURE ........................................................ CUTOFF

THROTTLE ...........................................................FULL CONTINUE CRANKING FOR A FEW SECONDS

FUEL SELECTOR .................................................. OFF MASTER SWITCH .................................................. OFF IGNITION SWITCH ................................................. OFF

EVACUATE AIRCRAFT, OBTAIN FIRE EXTINGUISHER, EXTINGUISH FIRE IF PRACTICAL

Vacuum System Failure Blockage    .  Rely on Turn Coordinator

_______________________________________________________________________

172 N Emergency & Abnormal                                                                                          3

Ammeter: Excessive Rate of Charge     .

Ammeter Full-Scale Deflection

ALTERNATOR ........................................................OFF NON-ESSENTIAL ELEC. EQUIPMENT .....................OFF

LAND AS SOON AS POSSIBLE

Compass may be off as much as 25 ̊

Over-Voltage Light Illuminates         .

MASTER SWITCH ................................OFF (Both sides) MASTER SWITCH ................................................... ON OVERVOLTAGELIGHT ................................CHECK OFF

IF OVER-VOLTAGE LIGHT ILLUMINATES AGAIN

LAND AS SOON AS POSSIBLE

Ammeter Shows Discharge               .

ALTERNATOR ..................................................OFF NONESSENTIAL ELEC. EQUIPMENT ...................OFF LAND AS SOON AS POSSIBLE

Low Voltage Light During Flight        .

RADIOS, AVIONICS POWER SWITCH .....................OFF MASTER SWITCH ...................................................OFF MASTER SWITCH ................................................... ON LOW VOLTAGE LIGHT ............................... CHECK OFF RADIOS, AVIONICS POWER SWITCH ….................. ON

IF LOW VOLTAGE LIGHT RE-ILLUMINATES

ALTERNATOR ........................................................OFF NON-ESSENTIALELECT.EQUIPMENT .....................OFF LAND AS SOON AS POSSIBLE

Static Source Blockage             .

ALTERNATE STATIC SOURCE VALVE ............. PULL ON AIRSPEED ............................................................  CONSULT TABLES IN POH/AFM SECTION 5

_______________________________________________________________________

172 N Emergency & Abnormal                                                                                          4

Engine Failure During Flight         .

AIRSPEED ...................................BEST GLIDE / 65 KIAS NOTE WIND DIRECTION AND SPEED

PICK & FLY TOWARDS LANDING SITE

CARB HEAT ............................................................ ON MIXTURE ............................................................. RICH FUEL SELECTOR  ..…………................................BOTH PRIMER ................................................... IN & LOCKED MAGNETOS .................................................CHECKALL

IF PROP NOT WINDMILLING

IGNITION SWITCH ..............................................START MAGNETOS ................................................CHECK ALL

Precautionary Landing with Engine Power    .

LANDING AREA .............................. SELECT & INSPECT RADIO & ELECTRICAL SWITCHES ..........................OFF

ON FINAL APPROACH

FLAPS .................................... (30 ̊Recommended) ASREQ MASTER SWITCH ...................................................OFF

DOORS ...............................(Prior to touchdown) UNLATCH IGNITION SWITCH ……............................................OFF BRAKES .............................................. APPLY HEAVILY

Emergency Landing No Engine Power    .

LANDING AREA ............................. SELECT & INSPECT AIRSPEED ..................... BEST GLIDE / 65 KIAS (Flaps Up) MIXTURE .................................................CUTOFF FUELSELECTORVALVE ..........................................OFF IGNITION SWITCH ……...........................................OFF FLAPS ........................ (40 ̊/60KIASRecommended)ASREQ MASTER SWITCH …….............................................OFF DOORS ...........................(Prior to touchdown) UNLATCH BRAKES .............................................. APPLY HEAVILY

_______________________________________________________________________

172 N Emergency & Abnormal                                                                                           5

Standardized Speeds

80 KIAS ........................................................FLAPS 10° - 1700 RPM 70 KIAS .............................................. FLAPS 20° - TURNING BASE

SHORT FINAL UNTIL 65 KIAS ...................10 to 20’ ABOVE RUNWAY

When landing assured

SOFT FIELD .........................................................65 KIAS - FLAPS 30° SHORT FIELD ............................................62 KIAS - FLAPS 30°

Pattern Work                     .

Touch & Go's Prohibited Except With Instructor

Before Landing Checklist

FUEL SELECTOR……............................................... BOTH MIXTURE …….............................................................. FWD CARB HEAT……..............................................................ON

Go Around / Missed Approach     .

THROTTLE …………....................................................FULL CARB HEAT .................................................................. OFF FLAPS ............................................ (If > 20°) RETRACT 20° PITCH ..............................................................BEST CLIMB FLAPS ........................................(> 55 KIAS) RETRACT 10° AIRSPEED ..............................................................VY / VX

AT SAFE ALTITUDE & > 65 KIAS

FLAPS……...................................................................... UP

FLY ASSIGNED/PUBLISHED HEADING & ALTITUDE

Touch and Go                    .

MAINTAIN CENTERLINE

FLAPS.............................................................................. UP CARB HEAT ..................................................................OFF THROTTLE ..................................................................FULL

After Landing                      .

TRIM ................................................................CENTERED CARB HEAT .................................................................. OFF FLAPS ............................................................................. UP LANDING & TAXI LIGHTS .......................................AS REQ MIXTURE ..................................................SLIGHTLY LEAN TAXI CLEARANCE ................................... OBTAIN & BRIEF

Before Takeoff                   .

––––––––––––––––––––

DOORS & WINDOWS .......................CLOSED & LOCKED LANDING LIGHT ………...........................................ON STROBE LIGHT .................................... (If installed) ON TRANSPONDER ......................................................ALT

FLAPS ....................................................................SET

MIXTURE ........................... FULL FWD (or set for altitude)

IPAD ............................................................................. OUT

HEADING INDICATOR…………………..………………..SET

______________________________________________ 172 Pattern Work                                                                                                         6